POCKET ADDIO

POCKET 8 INPUT/8 OUTPUT DIO BOX

POCKET DIO TERMINAL BOARD

OPERATION MANUAL

CONTENTS:

CHAPTER 1. Introduction

CHAPTER 2. Hardware Installation

CHAPTER 3. Hardware Configuration

CHAPTER 4. Software Programming

APPENDICES:

a. Warranty Information

b. Data Sheet

Rev. Date: 1996 JAN. 10

CHAPTER 1     INTRODUCTION

The  pocket  DIO is a digital input and digital  output  data

acquisition  box,  which  is  used  to  connect  to  print   port

(centronic)  of  PC/AT,  PC/386, PC/486,  Pentium  or  compatible

computer.  The  pocket DIO is a high precision  data  acquisition

system, it provides 8 digital input and 8 digital output lines.

The  pocket  DIO is no need to occupy any slot  of  your  PC

system,  it is suitable for any PC which contains parallel  print

port,  specially,  it is more useful for  notebooks  and  laptops

computers.

The features of the pocket DIO are:

* Support 8 digital input and 8 digital output lines.

* Digital input :

Logical 0 : 0.8V max, 2.0mA max.

Logical 1 : 2.0V min.

* Digital Output :

Logical 0 : 0.5V max, 24mA max.

Logical 1 : 1.2V min.

* Current consumption : 100 mA.

* Power adapter : DC 9 V, 500 mA.

The package contains:

* Pocket DIO.

* Power adapter.

* Expansion cable.

* User's manual.

* Pocket  DIO   terminal  board which  contains  8  channel  opto

isolated   input  and  8  channel  solid-state   relay   output

(optional).

CHAPTER 2      HARDWARE INSTALLATION

Your pocket DIO is designed to be inserted in parallel print

port of your computer, please follow the steps listed below:

1. Turn  off  all  power to your  computer  and  all  peripheral

devices before installing your pocket DIO.

2. Connect   expansion  cable  to parallel  printer  port,  then

insert pocket DIO to the connector of expansion cable.

3. Plug in power adapter to pocket DIO.

4. Turn on the computer and power adapter.

CHAPTER 3      HARDWARE CONFIGURATIONS

Before  you  use  the pocket DIO, you must  ensure  that  the

printer  port address and identifier address are  set  correctly.

The  proper  settings  for the pocket DIO are  described  in  the

following pages:

3.1 I/O Port address

The DIO signals are transferred via parallel printer port, so

that  the   I/O port address of pocket DIO are &H3BC,  &H378,  or

&H278 depends on your printer port setting.

3.2 LED Indicator

The  LED1 is used to indicate power active and LED2 is  used

to indicate signal access.

3.3 Identifier Address

Each  expansion  cable  contains  4  connectors  to  connect

maximum 4 pocket DIO or pocket A/D (you can mix it together), the

identifier address is used to identify the address of pocket DIO.

User  may  use  DIP switch to set identifier  address,  and  each

pocket DIO must be set to different identifier address.

Observe  figure above, to set the switch ON means 0 and  set

the switch OFF means 1. The figure above is set to 5.

3.4 Pin Assignment of Pocket DIO

Pin     Function       Pin    Function

1      GND            14     +5V

2      DI0            15     GND

3      DI1            16     GND

4      DI2            17     GND

5      DI3            18     GND

6      DI4            19     GND

7      DI5            20     DO3

8      DI6            21     DO4

9      DI7            22     DO5

10      DO0            23     DO6

11      DO1            24     DO7

12      DO2            25     +5V

13      GND

CHAPTER 4   SOFTWARE PROGRAMMING

In  the  distribution diskette, we provide  C,  PASCAL,  and

BASIC  examples to show how to write software to get  and  output

digital signals. It is easy to call getdi() and setdo() functions

to  get and set  the digital signal from C and PASCAL. For  BASIC

language, the functions are located at line number 1000.

1. Programming Guide

a. Initial

void ioinit(unsigned int addr);

addr    : printer port address.

b. Get digital signal

int getdi(int box);

box     : identifier address.

return  : digital value.

c. Set digital signal

void setdo(int box, int val);

box     : identifier address.

val     : set digital value.

2. Examples

The  examples  of  C, PASCAL, and BASIC  are  shown  in  the

following pages.

C PROGRAM

/* Pocket I/O Converter: Sample Program */

#include <stdio.h>

#include <dos.h>

#include <conio.h>

#include <time.h>

#include <ctype.h>

/* for test only */

#define TEST

#define LPT1 0x3BC

#define LPT2 0x378

#define LPT3 0x278

/* I/O ports for printer interface */

#define DATA(x)         (x + 0)

#define STATUS(x)       (x + 1)

#define CONTROL(x)      (x + 2)

/* definition of each data bit */

#define D7		0x80

#define D6		0x40

#define D5		0x20

#define D4		0x10

#define D3		0x08

#define D2		0x04

#define D1		0x02

#define D0		0x01

/* definition of status port */

#define BUSY	    	0x80

#define ACK         	0x40

#define PAPEROUT    	0x20

#define SELECTED    	0x10

#define ERROR       	0x08

/* definition of control port */

#define IRQENABLE	0x10

#define SELECT      	0x08

#define RESET       	0x04

#define LF          	0x02

#define STROBE      	0x01

/* pin assignments for Pocket I/O converter */

/* ADDR : box ID                            */

/* CLK  : clock for serial data in and out  */

/* DI   : data in (from box to host)        */

/* DO   : data out (from host to box)       */

/* EOC  : end of coversion                  */

#define	ADDR        	(D7 | D6 | D5 | D4)

#define LOAD		D3

#define DO		D2

#define CLK		D1

#define RW		D0

#define DI		SELECTED

static unsigned char port_value = 0;

static unsigned int port_addr = LPT1;

/* udelay : delay routine */

/* n      : loop count    */

static void

udelay (int n)

{

int i, j;

for (i = 0; i < n; i++)

for (j = 0; j < 10; j++);

}

/* ioinit : initial converter           */

/* addr   : I/O address of printer port */

void

ioinit(unsigned int addr)

{

port_addr = addr;

port_value = 0;

}

/* getdi   : get digital input value */

/* box     : box ID       	     */

int

getdi(int box)

{

int i, r;

port_value = (box << 4);

outportb (DATA(port_addr), port_value);

udelay (1);

outportb (DATA(port_addr), port_value | LOAD);

udelay (1);

outportb (DATA(port_addr), port_value);

udelay (1);

for (r = 0, i = 0; i < 8; i++) {

if (inportb (STATUS(port_addr)) & DI)

r |= 0x80 >> i;

outportb (DATA(port_addr), port_value | CLK);

udelay (1);

outportb (DATA(port_addr), port_value);

udelay (1);

}

return r;

}

/* setdo   : set digital output value */

/* box     : box ID          	      */

/* val     : output value             */

void

setdo(int box, int val)

{

int i;

port_value = (box << 4) | RW;

outportb (DATA(port_addr), port_value);

udelay (1);

for (i = 0; i < 8; i++) {

if (val & (0x80 >> i))

port_value |= DO;

else

port_value &= ~DO;

outportb (DATA(port_addr), port_value);

udelay (1);

outportb (DATA(port_addr), port_value | CLK);

udelay (1);

outportb (DATA(port_addr), port_value);

udelay (1);

}

outportb (DATA(port_addr), port_value | LOAD);

udelay (1);

outportb (DATA(port_addr), port_value);

udelay (1);

}

#ifdef TEST

void

dumpbin(unsigned int n)

{

int i;

for(i = 7; i >= 0; i--) {

cputs((n & (1 << i))? " ON  " : " OFF ");

}

}

main ()

{

int i, r, c, qflag;

ioinit (LPT1);

qflag = 0;

for (;!qflag;) {

r = getdi(0);

printf ("\r");

dumpbin(r);

if (kbhit()) {

c = getch ();

if (c == '0')

r = 0;

else if (c == '9')

r = 0xff;

else if (c >= '1' && c <= '8')

r =  1 << (c - '1');

else if (c == 27) {

r = 0;

qflag = 1;

}

setdo (0, r);

if (c >= '0' && c <= '9') {

printf (" Output : ");

if (c == '0')

printf ("  NONE ");

else if (c == '9')

printf ("  ALL  ");

else

printf (" CH. %d ", c - '0');

printf (" High");

}

}

}

clrscr ();

}

#endif

PASCAL PROGRAM

{ Pocket I/O Converter: Sample Program }

uses

Crt;

const

LPT1 = $3BC;

LPT2 = $378;

LPT3 = $278;

{ I/O ports for printer interface }

DATA    = 0;

STATUS  = 1;

CONTROL = 2;

{ definition of each data bit }

D7 = $80;

D6 = $40;

D5 = $20;

D4 = $10;

D3 = $08;

D2 = $04;

D1 = $02;

D0 = $01;

{ definition of status port }

BUSY     = $80;

ACK      = $40;

PAPEROUT = $20;

SELECTED = $10;

ERROR    = $08;

{ definition of control port }

IRQENABLE = $10;

SELECT    = $08;

RESET     = $04;

LF        = $02;

STROBE    = $01;

{ pin assignments for Pocket A/D converter     }

{ ADDR     : box ID                            }

{ CLK      : clock for serial data in and out  }

{ DATAIN   : data in (from host to box)        }

{ DATAOUTO : data out (from box to host)       }

{ EOC      : end of coversion                  }

IDMASK  = D7 or D6 or D5 or D4;

LOAD    = D3;

DATAOUT = D2;

CLK     = D1;

RW      = D0;

DATAIN  = SELECTED;

LPTTAB : array[1..3] of integer = (LPT1, LPT2, LPT3);

var

port_value : byte;

port_addr : integer;

lpt : integer;

addr : integer;

boxno, i, r : integer;

ch : char;

{ udelay : delay routine }

{ n      : loop count    }

procedure udelay (n : integer);

var

i,j : integer;

begin

for i := 1 to n do

for j := 1 to 10 do;

end;

{ ioinit : initial converter           }

{ addr   : I/O address of printer port }

procedure ioinit(addr : integer);

begin

port_addr := addr;

port_value := 0;

end;

{ getdi   : get digital input value }

{ box     : box ID       	    }

function getdi(box : integer) : integer;

var

i, r : integer;

begin

port_value := box shl 4;

port[DATA + port_addr] :=  port_value;

udelay(1);

port[DATA + port_addr] :=  port_value or LOAD;

udelay(1);

port[DATA + port_addr] :=  port_value;

udelay(1);

r := 0;

for i := 0 to 7 do

begin

if (port[STATUS + port_addr] and DATAIN) > 0 then

r := r or ($80 shr i);

port[DATA + port_addr] :=  port_value or CLK;

udelay(1);

port[DATA + port_addr] :=  port_value;

udelay(1);

end;

getdi := r;

end;

{ setdo   : set digital output value }

{ box     : box ID          	     }

{ val     : output value             }

procedure setdo(box : integer; val : integer);

var

i : integer;

begin

port_value := (box shl 4) or RW;

port[DATA + port_addr] := port_value;

udelay(1);

for i := 0 to 7 do

begin

if (val and ($80 shr i)) > 0 then

port_value := port_value or DATAOUT

else

port_value := port_value and not DATAOUT;

port[DATA + port_addr] :=  port_value;

udelay(1);

port[DATA + port_addr] :=  port_value or CLK;

udelay(1);

port[DATA + port_addr] :=  port_value;

udelay(1);

end;

port[DATA + port_addr] :=  port_value or LOAD;

udelay(1);

port[DATA + port_addr] :=  port_value;

udelay(1);

end;

{ dumpbin : print out ON/OFF status }

{ n       : value                   }

procedure dumpbin(n : integer);

var

i : integer;

begin

for i := 7 downto 0 do

if (n and (1 shl i)) > 0 then

write(' ON  ')

else

write(' OFF ');

writeln;

end;

begin

lpt := 1;

addr := LPTTAB[lpt];

boxno := 0;

ioinit (addr);

for i := 0 to 7 do

begin

setdo (boxno, 1 shl i);

r := getdi(boxno);

dumpbin (r);

ch := readkey;

end;

end.

BASIC PROGRAM

100   REM

110   REM           Pocket I/O Converter: Sample Program

120   REM

140   PV = 0

150   LPT1 = &H3BC : LPT2 = &H378 : LPT3 = &H278

160   PORT = LPT1

170   BOX = 0

180   FOR COUNT = 0 TO 7

190     DOUT = 2 ^ COUNT

200     GOSUB 1500

210     GOSUB 1000

220     FOR J = 0 TO 7

230       IF (DIN AND (2 ^ J)) > 0 THEN PRINT " ON  "; ELSE PRINT " OFF ";

240     NEXT

250     PRINT

260     FOR K = 1 TO 10000 : NEXT

270   NEXT

999   END

1000  REM      GET I/O VALUE

1010  REM      INPUT :

1020  REM           BOX : BOX ID

1040  REM      OUTPUT :

1050  REM           DIN : DIGITAL INPUT VALUE

1060  REM ===========================================

1070  PV = BOX * 16

1080  OUT PORT, PV

1090  OUT PORT, PV OR &H8

1100  OUT PORT, PV

1110  DIN = 0

1120  FOR I = 0 TO 7

1130    X = INP (PORT + 1) AND &H10

1140    IF X > 0 THEN DIN = DIN OR (2 ^ (7 - I))

1150    OUT PORT, PV OR &H2

1160    OUT PORT, PV

1170  NEXT

1180  RETURN

1500  REM      SET I/O VALUE

1510  REM      INPUT :

1520  REM           BOX  : BOX ID

1550  REM           DOUT : DIGITAL OUTPUT VALUE

1560  REM ===========================================

1570  PV = BOX * 16 + &H1

1580  OUT PORT, PV

1590  FOR I = 0 TO 7

1600    IF (DOUT AND (2 ^ (7 - I))) > 0 THEN PV = PV OR &H4 ELSE PV = PV AND &HFB

1610    OUT PORT, PV

1620    OUT PORT, PV OR &H2

1630    OUT PORT, PV

1640  NEXT

1650  OUT PORT, PV OR &H8

1660  OUT PORT, PV

1670  RETURN

APPENDIX  A  WARRANTY INFORMATION

A.1 Copyright

Copyright 1996 DECISION COMPUTER INTERNATIONAL CO., LTD. all

rights reserved. No part of POCKET ADDIO software and manual  may

be  reproduced, transmitted, transcribed, or translated into  any

language  or  computer  language, in any form or  by  any  means,

electronic,  mechanical, magnetic, optical, chemical, manual,  or

otherwise,  without  the  prior written  permission  of  DECISION

COMPUTER INTERNATIONAL CO.,  LTD.

Each  piece  of  POCKET ADDIO package permits  user  to  use

POCKET  ADDIO  only on a single computer, a registered  user  may

use  the  program on a different computer, but may  not  use  the

program on more than one computer at the same time.

Corporate   licensing  agreements  allow   duplication   and

distribution  of  specific number of copies within  the  licensed

institution. Duplication of multiple copies is not allowed except

through  execution  of a licensing agreement.  Welcome  call  for

details.

A.2 Warranty Information

POCKET ADDIO warrants that for a period of one year from the

date  of  purchase (unless otherwise specified  in  the  warranty

card)  that  the  goods supplied will perform  according  to  the

specifications defined in the user manual.  Furthermore that  the

POCKET  ADDIO   product  will be supplied free  from  defects  in

materials  and workmanship and be fully functional  under  normal

usage.

In the event of the failure of a POCKET ADDIO product within

the specified warranty period, POCKET ADDIO will, at its  option,

replace or repair the item at no additional charge. This  limited

warranty  does  not cover damage resulting  from  incorrect  use,

electrical   interference,  accident,  or  modification  of   the

product.

All goods returned for warranty repair must have the  serial

number intact.  Goods without serial numbers attached will not be

covered by the warranty.

Transportation costs for goods returned must be paid by  the

purchaser.   Repaired goods will be dispatched at the expense  of

POCKET ADDIO.

To  ensure that your POCKET ADDIO product is covered by  the

warranty provisions, it is necessary that you return the Warranty

card.

Under this Limited Warranty, POCKET ADDIO's obligations will

be  limited to repair or replacement only, of goods found  to  be

defective as specified above during the warranty period.   POCKET

ADDIO is not liable to the purchaser for any damages or losses of

any  kind,  through the use of, or inability to use,  the  POCKET

ADDIO product.

POCKET   ADDIO   reserves  the  right  to   determine   what

constitutes warranty repair or replacement.

Return Authorization: It is necessary that any returned goods are

clearly  marked with an RA number that has been issued by  POCKET

ADDIO.   Goods  returned without this authorization will  not  be

attended to.

APPENDIX  B  DATA SHEET

POCKET ADDIO

POCKET DIO TERMINAL BOARD

CONTENTS:

CHAPTER 1. Pocket DIO Terminal Board

(8 relay and 8 photo isolator)

APPENDIX:

APPENDIX A   Warranty Information

Rev. Date: 1996 JAN. 28

CHAPTER 1   POCKET DIO TERMINAL BOARD

The 8 channels relay output / photo isolator input board for

pocket  DIO  provides relay output functions and  photo  isolator

input  functions.  The relay output part provides 8  Single  Pole

Single Throw (SPST) relays to drive 8 different output  channels.

Each  relay channel can be used to control ON / OFF  of  external

devices, to drive external high power relays, to activate  alarms

... etc.

The  photo  isolator  input part  provides  8  photo  couple

digital  input  channels,  which allow the input  signals  to  be

completely floated and prevent the ground loop.

The  features of 8 channels relay output / 8 channels  photo

isolator input board are:

* Support  8  relay  output channels and 8  photo  couple  input

channels.

* Max  contact rating for relay: 120V AC/DC   1AMP.

* Attraction time for relay: 3 ms.

* Fall off time for relay: 2 ms.

* Isolation resistance for relay: 100M OHM.

* Life  expectancy  for relay:  5  million    operation  at  full

load.

* The  Normal Open (NO)  and Common contacts (COM) of each  relay

is brought out to the     screw connector.

* Breakdown voltage : AC/DC 500V minimum.

* Allow the  photo input  signals to be   completely floated  and

prevent the ground   loops.

* Input current :  80mA  maximum for each   isolated input.

* Input voltage  : 15V DC  maximum for each   isolated input.

* LED  indicates when corresponds channel is   activated.

* Isolated or non-isolated modes selectable.

1. Terminal Board

The  figure  of pocket DIO terminal board is  shown  in  the

follows.

2. Hardware Configuration

Before you use the  terminal board, you must ensure to  plug

in this board to pocket DIO.

The LED16 in terminal board shows the power on status.  When

it turns to light, we have power to this board, otherwise when it

turns to dark, no power is provided.

The Jumpers are used to select isolated or non-isolated mode,

when  open the jumpers, it sets to isolated mode, otherwise  when

short the jumpers, it sets to non-isolated mode. JP0 and JP1  are

correspond to channel 0, JP2 and JP3 are correspond to channel 1,

etc.   When set to non-isolated mode, the IxL is short to  system

ground. (where x is the channel number)

3. Pin Assignment of T1

Pin  Description      Pin  Description

1      GND           14      +5V

2      DI0           15      GND

3      DI1           16      GND

4      DI2           17      GND

5      DI3           18      GND

6      DI4           19      GND

7      DI5           20      DO3

8      DI6           21      DO4

9      DI7           22      DO5

10      DO0           23      DO6

11      DO1           24      DO7

12      DO2           25      +5V

13      GND

4. Pin Assignment of TB1 (Photo Isolator)

Pin   Description

1      I0H

2      I0L

3      I1H

4      I1L

5      I2H

6      I2L

7      I3H

8      I3L

9      I4H

10      I4L

11      I5H

12      I5L

13      I6H

14      I6L

15      I7H

16      I7L

5. Pin Assignment of TB2 (Relay Output)

Pin   Description

1      COM0

2      OTO0

3      COM1

4      OTO1

5      COM2

6      OTO2

7      COM3

8      OTO3

9      COM4

10      OTO4

11      COM5

12      OTO5

13      COM6

14      OTO6

15      COM7

16      OTO7

APPENDIX A   WARRANTY INFORMATION

A.1 Copyright

Copyright 1996 DECISION COMPUTER INTERNATIONAL CO., LTD. all

rights reserved. No part of POCKET ADDIO software and manual  may

be  reproduced, transmitted, transcribed, or translated into  any

language  or  computer  language, in any form or  by  any  means,

electronic,  mechanical, magnetic, optical, chemical, manual,  or

otherwise,  without  the  prior written  permission  of  DECISION

COMPUTER INTERNATIONAL CO.,  LTD.

Each  piece  of  POCKET ADDIO package permits  user  to  use

POCKET  ADDIO  only on a single computer, a registered  user  may

use  the  program on a different computer, but may  not  use  the

program on more than one computer at the same time.

Corporate   licensing  agreements  allow   duplication   and

distribution  of  specific number of copies within  the  licensed

institution. Duplication of multiple copies is not allowed except

through  execution  of a licensing agreement.  Welcome  call  for

details.

A.2 Warranty Information

POCKET ADDIO warrants that for a period of one year from the

date  of  purchase (unless otherwise specified  in  the  warranty

card)  that  the  goods supplied will perform  according  to  the

specifications defined in the user manual.  Furthermore that  the

POCKET  ADDIO   product  will be supplied free  from  defects  in

materials  and workmanship and be fully functional  under  normal

usage.

In the event of the failure of a POCKET ADDIO product within

the specified warranty period, POCKET ADDIO will, at its  option,

replace or repair the item at no additional charge. This  limited

warranty  does  not cover damage resulting  from  incorrect  use,

electrical   interference,  accident,  or  modification  of   the

product.

All goods returned for warranty repair must have the  serial

number intact.  Goods without serial numbers attached will not be

covered by the warranty.

Transportation costs for goods returned must be paid by  the

purchaser.   Repaired goods will be dispatched at the expense  of

POCKET ADDIO.

To  ensure that your POCKET ADDIO product is covered by  the

warranty provisions, it is necessary that you return the Warranty

card.

Under this Limited Warranty, POCKET ADDIO's obligations will

be  limited to repair or replacement only, of goods found  to  be

defective as specified above during the warranty period.   POCKET

ADDIO is not liable to the purchaser for any damages or losses of

any  kind,  through the use of, or inability to use,  the  POCKET

ADDIO product.

POCKET   ADDIO   reserves  the  right  to   determine   what

constitutes warranty repair or replacement.

Return Authorization: It is necessary that any returned goods are

clearly  marked with an RA number that has been issued by  POCKET

ADDIO.   Goods  returned without this authorization will  not  be

attended to.

################################################################